GCS 235
[image: image1.jpg]



Official #:


1214462

Year Built:


November, 2008
Dimensions:


300 L x 64 W x 21 D (Molded)
Draft Loaded:


14.5 Ft. @ 95% Capacity
Cargo Tank Capacity:

38,880 Bbls. @ 100%




36,860 Bbls. @ 95%





Cargo tanks 1/8” Increments





Five (5) Cargo Tanks Port/Starboard





Ten (10) Compartments
Notch:



Yes, Large Capacity
Product Grade:

(A) and All lower grades
Classification:


ABS Class Certificate +A1 Oil Tank




Barge, Oceans, Coastwise, Manned, US & International 

Tonnage
Cargo Systems:

Multi Grade with Double Block Valves
Load/Discharge:

Two Grades Simultaneously
Manifold:


Two 8" Cargo Hoses, One 8" Chicksan, Two 8" Vapor 

Recovery, Located Port and Starboard Sides
High Level Alarm System:
Bergen Design System - Closed Gauging
Cargo Pumps (2):

(2) Byron Jackson Three Stage Pumps Model 12-LS-16-

GH - 4,000 Bbls. p/h
Pump Engines (2):

Tier Two Certified John Deere Model 6125A 400 HP
Generators (2):

Tier Two Certified John Deere Model 4045 TMGK, 65 

KW, 480 Volt Marathon Model 362PDL1604
Cargo Deck Lighting (12):
400W Metal Halide Flood Lights
Vapor Recovery System:
8" Vapor Lines
Air System (1):

Quincy Model 325QRB with 5HP Motor and 80 Gallon 

Receiver
Anchor System:

4,000 Lb. Stockless Anchor with Coastal Marine 
Equipment MO lA20-137B3-135-00 Anchor Hydraulic 
Winch
Diesel Fuel Tank:

Capacity 6,000 Gallons with High Level Alarm, 

Mechanical Site Glass with Containment
Potable Water Tank:

Capacity 2,100 Gallons Epoxy Coated
Holding Tank:


Capacity 1,000 Gallons Epoxy Coated for Grey/Brown 

Water to meet ZERO (0) Discharge Regulations 
Cargo Hose Crane:

4,000 Lbs. SWL with 65' Radius Hyd.

Capstan:


(1) Hydraulic 18" Dia. - Coastal Marine Equipment MO. 

CI7-15-63-11 15,000 # @ 30 Ft. Per Minute

Paint Coatings:

Epoxy Coated - Hull, Voids, Deck & Cargo Tks.

Crew Quarters:

(4) Berths with Shower/Toilet and Galley with AC and 

Heat

Deck Containment:

12" H x YS" Flat bar Around Entire Perimeter of Tank 

Barge

Storage:


(1) 4.6'W x 8'H x 10'L House

(4) 3'W x 5'L x 3'H Covered Storage Boxes 

Forepeak 12'W x 24'L Galvanized Grating Platform 

Aft Peak 16'W x 24'L Galvanized Grating Platform

....
